Barbara Dill

http://www.youtube.com/watch?feature=player_embedded&v=JK1EvP_NCI0

Materials

Wood: Size; Min. 2x2x6”; Max. 6X6X?” (Goblet sizes).

Use pencil and then Sharpie to mark sides and dots.

Compass

Be accurate in the markings.

Process:

- Place Blank between centers:

      - Round the blank.

      - Make a tenon on both ends; these are reference point as a depth gauge for the turning.

        - Bottom is usually larger than the top for stability.

      - Mark the position for the handle or grasping area on the head stock side. (Usually about 1/3 from finished bottom or 2/3 total length from the top.

- Marking the 120 degree points:

      - Using the compass mark 3 points at 120 degrees angles from each other.

        - Mark both tenons. 1-1; 2-2; 3-3.

        - You now have a large Y shape on each end marked. (Make a template).

        - On each line mark the hole, so that it does not interfere with the center drive (get a full bite with the center drive).

      - Another method is to use the indexing on the lathe.

        - A twelve-position index has 30 degrees per indent.

        - Mark positions 1, 5 and 9 for the 120-degree lines.

        - Use the tools rest to transfer the marks from headstock to tailstock.

        - Label the lines and set the marks.

- After marking;

      - Prepress the Marks on the lathe. 1–2; 2-3, 3-1.

      - Do not use a punch or awl they produce a straight hole.

- Position A: Headstock Mark # 1; Tailstock Mark # 3.

      - Round down to just touching the tenon on each end.

      - Make a cove (or a v cut) for the handle using the reference line made earlier.

      - Clean cuts and the sand (slow speed).

      - Stop often and check your progress.

- Position B: Headstock Mark # 2; Tailstock Mark # 1.

      - Round down to just touching the tenon on each end.

      - Make a cove (or a v cut) for the handle using the reference line made earlier.

      - Clean cuts and the sand (slow speed).

      - Note: Stop often and check your progress.

- Position C: Headstock Mark # 3; Tailstock Mark # 2.

      - Round down to just touching the tenon on each end.

      - Make a cove (or a v cut) for the handle using the reference line made earlier.

      - Clean cuts and the sand (slow speed).

      - Note: Stop often and check your progress.

- Position D:

      - Between centers: make the headstock tenon available for a 4-jaw chuck.

      - Refine the top and bottom edges of the cup.

        - Using the spindle gouge.

- Position E:

      - Place in a 4 jaw chuck.

      - Hollow.

      - Sand.

- Finishing:

      - Seal with THIN CA Glue.

        - Use hemostat and small balls of paper towel to get even coating.

        - CA glue the lip edge (end grain).

      - Dry for 24 hours.

        - Sand with 400 grit.

        - Recoat with CA glue.

        - Repeat until satisfied with finish.